University of the People

CS 2204-01 Communications and Networking - AY2025-T1

Written Assignment 6

10 October 2024

- IPv4 Fragmentation for 2000 Bytes over UDP

When transmitting 2,000 bytes of user data via a single UDP send with a standard 1,500-byte Ethernet payload, we must account for both the IP and UDP headers. The IP header is 20 bytes, and the UDP header is 8 bytes, leaving 1,472 bytes for data per fragment.

To transfer 2,000 bytes:

First fragment: 1,472 bytes of data + 28 bytes (IP + UDP headers) = 1,500 bytes.

Second fragment: The remaining 528 bytes of data.

Thus, 2 fragments are needed to transmit 2,000 bytes of data: the first with 1,472 bytes and the second with 528 bytes.

- Problems with RPC (Remote Procedure Call)

Despite its conceptual elegance, RPC has several problems:

- Transparency issues: While RPC aims to make remote calls appear like local ones, differences between local and remote environments (latency, failure, etc.) can create issues.

- Error handling: Handling remote server crashes or network failures is complex, as the client may remain unaware of whether a call succeeded or failed.

- Performance overhead: Serialization, deserialization, and network transmission introduce delays and overhead compared to direct function calls.

- Timestamping in Real-Time Applications (RTP)

In real-time applications, timestamping is crucial for maintaining proper synchronization of media streams. It helps in:

- Synchronizing playback by ensuring that packets arrive in the correct sequence.

- Managing jitter by compensating for variable packet delays.

- Determining when data is valid for playback, which is essential in maintaining audio or video quality.

- Why UDP Exists

UDP exists because it provides a lightweight, connectionless protocol that offers fast transmission for applications that do not require reliable delivery, such as streaming media, DNS, and real-time applications. Letting user processes send raw IP packets would bypass essential services like port multiplexing and checksums, which UDP provides.

- QUIC and Reduced RTTs

QUIC, designed for modern web protocols, reduces the round-trip time (RTT) at the start of a secure connection by combining the transport layer and TLS handshake into a single step. It allows the initial packet to start the secure handshake and data transmission simultaneously, eliminating multiple RTTs that are usually required in traditional TCP + TLS setups